Summary of 2019/ 20 Ballast Exposure Monitoring Programme

| Work Activity: | Weather: | Ballast condition: | Personnel Monitored: | Static Monitoring: | Conclusion: | Recommendations: |
| --- | --- | --- | --- | --- | --- | --- |
| Stone blowing | Wet & Windy | Wet | 2x TQS | At various points around the machine | Low risk of ballast dust to the TQS: - Natural air movements around the machine - Short shift duration of 1-2hrs - The position of TQS in relation to the machine | NO Respiratory Protective Equipment (RPE) required for this work activity undertaken for similar duration. *RPE should be available on request |
|  | Dry & warm | Dry ballast | 1x TQS | At various points around the machine |  |  |
| Parallel Stone blowing | Dry with very little wind | Dry ballast | 1x TQS | At various points around the machine | Low risk of ballast dust to the TQS: - Natural air movements around the machine - Short shift duration of 1-2hrs - The position of TQS in relation to the machine | NO Respiratory Protective Equipment (RPE) required for this work activity undertaken for similar duration. *RPE should be available on request |
| Ballast dropping (Train) | Dry | Dry ballast | 1x Wagon Operator 1x Coss | At various points between wagons | Highest risk to Wagon Operator role due to proximity to drop sites Personal samples did not exceed exposure limit due to: - Natural air movements around the machine - Short shift duration of 1-2hrs - Personnel operating a distance away from the machine - Ballast was dropped in a controlled manor | Respiratory Protective Equipment (RPE) required for wagon operator role and anyone less than a 1m from the drop sites. |
|  | Dry with light breeze | Dry ballast | 1x Wagon Operator 1x Coss | At various points between wagons |  |  |
| Work Activity: | Weather: | Ballast condition: | Personnel Monitored: | Static Monitoring: | Conclusion: | Recommendations: |
| Ballast brushing/ Mechanical reprofiling of ballast | Moderate rain & significant wind | Damp ballast | 1x Machine controller 1x Machine Operator | Front and rear of ballast brusher unit and front of RRV (machine transfer shift only) | Personal samples did not exceed exposure limit due to: - Natural air movements around the machine - Short shift duration of 1-2hrs - Personnel operating a distance away from the machine - Wet weather provides natural dust suppression | Respiratory Protective Equipment (RPE) required by machine controller. |
|  | Light rain & moderate winds | Damp ballast | 1x Machine controller 1x Machine Operator | Front and rear of ballast brusher unit and front of RRV |  |  |
|  | Dry with moderate winds | Damp ballast | 1x Machine controller 1x Machine Operator | Front of ballast brusher unit, side of track and front of RRV |  |  |
|  | Dry with moderate wind | Damp ballast | 1x Machine controller 1x Assessor | Front and rear of ballast brusher unit and cab of RRV |  |  |
| Lifting and Packing | Dry & moderate wind | Dry ballast | 4x Operatives 1x Hygienist | None | Personal samples did not exceed exposure limit due to: - Natural air movements around the work activity - Short duration of work 1-2hrs | NO Respiratory Protective Equipment (RPE) required for this work activity undertaken for similar duration. *RPE should be available on request |
|  | Dry with light breeze | Dry ballast | 10x Operatives | None |  |  |
| Lifting and Packing (Red Zone) | Dry & moderate breeze | Dry ballast | 1x Supervisor 3x Operatives | None | Minimal personal exposure due to: - Short duration of work - Natural air movement around the work activity *Train movements did not affect dust levels on site during this shift. | No Respiratory Protective Equipment (RPE) required for this work activity undertaken for similar duration. *RPE should be available on request |
| Work Activity: | Weather: | Ballast condition: | Personnel Monitored: | Static Monitoring: | Conclusion: | Recommendations: |
| Track Inspection (Red Zone) | Dry | N/A | 1x Assessor 4x Operatives | None | Minimal personal exposure results due to: - No ballast was disturbed during this activity | No Respiratory Protective Equipment (RPE) required for this work activity. |
| Manual Reprofiling of ballast | Dry | Dry ballast | 4x Operatives | None | Minimal personal exposure due to: - Short duration of work 1-2hrs - Natural air movement around the work activity | No Respiratory Protective Equipment (RPE) required for this work activity. *RPE should be available on request |
| Manual Wet Bed Ballast removal | Dry | Damp ballast | 7x Operatives | None | Minimal personal exposure risk due to: - Short duration of work activity 1- 2hrs - Ballast was damp | No Respiratory Protective Equipment (RPE) required for this work activity. *RPE should be available on request |
|  | Dry & light breeze | Damp ballast | 7x Operatives | None |  |  |
|  | Spots of rain & light wind | Damp ballast | 7x Operatives | None |  |  |
|  | Heavy rain & strong winds | Damp ballast | 7x Operatives | None |  |  |

Key points:

Further exposure monitoring will be undertaken for those work activities where rain and damp ballast condition prevented a “worst case scenario” assessment. This will be delivered in Summer 2020/ 21.

The current recommendation does not require Respiratory Protective Equipment (RPE) to be mandated when undertaking several of the work activities above, however, good practise advise that employees are provided with the option/ choice to use disposable masks should they wish.

Any employees that want to use disposable masks, will need to be face-fit-tested and need to be clean shaven to use.

In Summary: (The below is from Richard Bunn – STE)

When outside, clouds of released dust are very quickly dispersed into the surrounding atmosphere. In a Network Rail survey of 11 sites, it was found that working more than 5m from ballast dust is unlikely to present a need for RPE. By 10m the HSE has found that levels around work sites are not usually higher than ambient air levels.

If the air is visibly dusty, if dust is detectable or if conditions are particularly hot and dusty, RPE should be worn.